# Clause 7.1.5 – Monitoring & Measuring Resources: Vehicle WA75 YZJ

## 1. Clause-Mapped Compliance Statement

This document outlines the monitoring and measuring resources maintained for Vehicle WA75 YZJ. It satisfies the requirements of ISO 9001:2015 and ISO 45001:2018 Clause 7.1.5 by ensuring that tools used for operational tasks are suitable, maintained, and monitored to ensure reliable performance and safe use. Vibration exposure is monitored using HAVS tracking sheets, and tools are stored securely within the vehicle with controlled access.

## 2. Vehicle-Specific Tool Register – Vehicle WA75 YZJ

| Tool Name | Model/Type | Typical Vibration (m/s²) | HAVS Risk Category | Purpose | Storage Location | Photo Reference |
| --- | --- | --- | --- | --- | --- | --- |
| Hilti TE 6-22 | Hammer Drill | 6–9 | Medium | Masonry drilling | Vehicle Tool Vault | Photo on file |
| Husqvarna 135 | Chainsaw | 6–10 | Medium | Cutting/sectioning | Vehicle Tool Vault | Photo on file |
| Hilti SID 6-22 | Torque Wrench | 2–4 | Low | Bolt tightening | Vehicle Tool Vault | Photo on file |
| Hilti GX 90-WF | Nail Gun | 2–4 | Low | Fixing/nailing | Vehicle Tool Vault | Photo on file |
| Bosch GSH Hammer Drill | Hammer Drill | 6–15 | High | Masonry breaking/drilling | Vehicle Tool Vault | Photo on file |

## 3. HAVS Exposure Sheet – Vehicle WA75 YZJ

| Tool | Vibration Magnitude (m/s²) | Trigger Time (mins) | Points | Recommended Individual Use (mins/day) | Notes |
| --- | --- | --- | --- | --- | --- |
| Hilti TE 6-22 | 6–9 Med |  |  | 30–60 mins/day | Masonry drilling |
| Husqvarna 135 | 6–10 Med |  |  | 30–60 mins/day | Cutting/sectioning |
| Hilti SID 6-22 | 2–4 Low |  |  | 60–120 mins/day | Bolt tightening |
| Hilti GX 90-WF | 2–4 Low |  |  | 60–120 mins/day | Fixing/nailing |
| Bosch GSH Hammer Drill | 6-15 High |  |  | 10-30 mins/day | Masonry breaking/drilling |

## 4. Monitoring & Measuring Controls

Tools are visually inspected before and after use for signs of damage or wear.

HAVS exposure is tracked daily using the provided exposure sheet.

Batteries are charged using manufacturer-approved chargers and stored safely in the vehicle.

Tools are stored in a secure vehicle tool vault with controlled access.

Any tool found to be defective is removed from service and reported for repair or replacement.

## 5. Records Retained

Tool Register (Vehicle Tool Vault)

HAVS Exposure Sheets (Daily, retained for 5 years)

Vehicle Inspection Sheets (Monthly, retained for 2 years)

Tool Issue/Return Logs (Retained for 1 year)

Battery Maintenance Records (Retained for 1 year)

# 6. HAVS Figures for CAT & GENNY

There is no HAVS exposure figure, trigger time, or vibration magnitude assigned to a CAT & GENNY system.

### Why?

- A CAT (Cable Avoidance Tool) and GENNY (Signal Generator) are not vibrating tools.

- They do not produce hand-arm transmitted vibration, so they fall outside the scope of the Control of Vibration at Work Regulations for exposure calculation.

- They therefore have no m/s² value, no exposure points, and no A(8) contribution.